Furniture utilities for AutoCAD.

mbox command (Flat detail): 
Draw flat detail with specified thickness. (thickness you can specify by mtolsh command). You select two points (parallelepiped). The detail will create in plane, correspondence with two max sizes of the parallelepiped. By default thickness is 16mm. 

mbox2 command (Flat detail XY):
Draw flat detail with specified thickness. (thickness you can specify by mtolsh command). You select two points. The detail will create in XY plane. By default thickness is 16mm. 

munion (group union),msubtract (group subtract),mintersect (group intersect) commads:
Union, subtraction and intersection detail with its copy, moving by some distance. The distance you can specify like using copy command. It is very convenient for edit overall sizes of models. For example, quick reduce the depth of a wardrobe.

mtolsh command (Flat detail thickness): 
Allow to specify the thickness of a flat detail (for using with mbox and mbox2 commands). 

mvirez command: 
Subtract selection1 from each detail in selection2.

mvirez2 command:
Subtract all details in drawing from each detail in selection. 

mplosh command: (Area):
Calculate total area of the all flat details in drawing.

mploshsel command:
Calculate total area of the selected flat details.

copyxy command:
Copying details like copy command, but displacement only in XY plane.

movexy command:
Moving details like move command, but displacement only in XY plane.

mpoly command:
Create polyline from some lines and arcs.

Mcopyarr command:

Copy details evenly between two points.